# METHOD OF STATEMENT

Date: 13th Mar 2025

Description of work

To shift out machine from workshop to loading area, hoist up onto lorry chassis, transport to new location, unload, shift and position into workshop

Machine Spec

1 unit Lead Test Cabinet

1 unit MPI machine

1 unit MPI cabinet

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/03/25

Time of Operation

09:45 am

Location of Operation

Singapore Institute of Technology

#### Move out: 10 Dover Dr

#### Move in: 13 New Punggol Rd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Crowbar/woodblock

Wire/webbing sling

PVC/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location

- Shift out machine from production to loading area

- Rigger to insert sling to crane hook and secured onto rigging point of machine

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to new location

- Hoist down machine from lorry chassis

- Insert pallet truck onto machine bed

- Shift and transport machine into workshop at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175