# METHOD OF STATEMENT

Date: 28th August 2020

Description of work

To shift out 1 unit Receiver Tank

Machine Spec

Machine Dimension: (Dia 80cm x height 260cm)

Machine Weight: 400kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/08/2020

Time of Operation

09:45 am

Location of Operation

#### 16 Joo Koon Crescent

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift out machine from production to loading area

- Attached wire sling to crane hook and secure to machine hoisting point

- Hoist up machine about 1ft above ground to check on balancing

- When well balance, hoist up machine unload on ground floor

- Shift and position into premises.

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175