# METHOD OF STATEMENT

Date: 08th May 2025

Description of Work

To unload 5 cases of 1 unit machine from 3 x 40ft FR and 1 x 20ft GP container, uncrate case, shift and position machine into production area at ground floor

Machine Spec

Variaxis I-1050T

Case 1: 743 x 368 x 331 cm   G/N Wt: 23, 080 kg / 19, 000 kg

Case 2: 562 x 329 x 359 cm   G/N Wt: 15, 571 kg / 13, 500 kg

Case 3: 445 x 161 x 293 cm   G/N Wt: 2, 980 kg / 2, 210 kg

Case 4: 518 x 228 x 225 cm   G/N Wt: 2, 680 kg / 1, 700 kg

Case 5: 221 x 150 x 130 cm   G/N Wt: 650 kg / 330 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/05/25

Time of Operation

10:00 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 70ton mobile crane

1-2 x 633 lorry loader

1 x short tail low bed

1 x forklift

Rollers/pallet truck

Manual/hydraulic jack

Wire/webbing sling

Shackle

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane to near container for the lifting work

- Set up crane with outrigger fully extended

- Insert sling below machine case

- Hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Load machine onto low bed and transport to door entrance

- Hoist down and assemble machine parts

- Jack up and insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175