# METHOD OF STATEMENT

Date: 22nd Dec 2025

Description of work

To shift out chiller from ground floor, hoist up to 4th floor building roof top opening by using mobile crane

Spec

2 units Chiller

Wt: 3, 800 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27/12/25

Time of Operation

09:00 am

Location of Operation

#### 14 Senoko Way

Equipment Use

1 x 50ton mobile crane

1 x 633 lorry loader

Wire sling

Webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety Shoe

Safety Helmet

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on metal plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- Shift out chiller from ground floor to hoisting area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of chiller

- Hoist up 1ft above ground to check on balancing

- When well balanced, to hoist up load to 4th floor rooftop opening

- Lower down chiller to near ground level

- Insert rollers under chiller bed

- Shift and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175