# METHOD OF STATEMENT

Date: 08th Sep 2021

Description of Work

To unload 8 cases of 2 unit machine from 1 x 40ft FR and 2 x 40ft HQ container, uncrate case, shift and position into production area at ground floor

Machine Spec

1 unit Koyo Centerless Grinding

Case 1: 330 x 310 x 250 cm / Wt: 8, 000 kg

Case 2: 330 x 228 x 195 cm / Wt: 900 kg

Case 3: 270 x 210 x 225 cm / Wt: 900 kg

Case 4: 470 x 228 x 215 cm / Wt: 800 kg

Case 5: 400 x 228 x 150 cm / Wt: 1, 800 kg

1 unit Koyo Grinding / HD5S

Case 1: 370 x 280 x 255 cm / Wt: 7, 400 kg

Case 2: 380 x 200 x 165 cm / Wt: 650 kg

Case 3: 300 x 210 x 220 cm / Wt: 1, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/09/21

Time of Operation

09:45 am

Location of Operation

#### 1 Gul Way

Meritor Heavy Vehicle Systems (S) Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Manual/hydraulic jack

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area for the lifting

- Insert sling below machine case and attached onto crane hook

- Hoist down case from container to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane and lower down machine near door entrance

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175