# METHOD OF STATEMENT

Date: 20th Sep 2024

Description of work

To shift out 1 machine from CMM room to loading area, hoist up onto lorry chassis and deliver to scrapyard for disposal

Equipment Spec

1 unit 7107 CMM

Dim: 700 x 1000 x 1200 mm

Wt: 1, 100 kg

Risk Assessment

A copy of Risk Assessment will be attach

Period of Operation

Start date: 01/10/24

End date: 01/10/24

Time of Operation

Start time: 09:30 am

End time: 11:30 am

Location of Operation

#### 1 Pandan Crescent

#### Emerson

Equipment Use

1 x lorry crane

Pallet truck

Canvas/PVC (to lay out for floor protection)

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine from CMM room

- Transport machine to loading area

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry crane to hoist up and load machine onto lorry chassis

- Release sling from rigging point of machine

- Secured with strapping belt prior to moving out of premise

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175