# METHOD OF STATEMENT

Date: 29th Jan 2024

Description of Work

To unload 13 cases of 1 unit machine and accessories from 2 x 40ft FR, 1 x 20ft FR, 2 x 40ft HQ and 1 x 20ft container, uncrate case, shift and position into production area at ground floor

Machine Spec

Horizontal Machining Center / HCN 10800

Case 1: 4, 250 x 2, 850 x 3, 580 / Wt: 13, 800 kg

Case 2: 4230 x 2760 x 1630 / Wt: 11, 500 kg

Case 3: 2450 x 1600 x 3430 / Wt: 5, 000 kg

Case 4: 5240 x 3390 x 3490 / Wt: 10, 800 kg

Case 5: 3710 x 2150 x 3630 / Wt: 6, 200 kg

Case 6: 3900 x 2150 x 3210 / Wt: 1, 700 kg

Case 7: 4450 x 2250 x 2180 / Wt: 2, 200 kg

Case 8: 2960 x 2250 x 1750 / Wt: 1, 250 kg

Case 9: 3500 x 2270 x 2460 / Wt: 1, 400 kg

Case 10: 3460 x 2240 x 2250 / Wt: 1, 600 kg

Case 11: 6360 x 2010 x 1900 / Wt: 2, 500 kg

Case 12: 3500 x 2200 x 1900 / Wt: 2, 300 kg

Case 13: 2000 x 1000 x 700 / Wt: 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 1 Seletar Aerospace Crescent

Rolls-Royce Singapore Pte Ltd

Equipment Use

1 x 50ton mobile crane

633 lorry loader

1 x forklift

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Hydraulic jack

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Setting up of crane with outrigger fully extended and resting on metal plate as base plate

- Insert sling under case

- Mobile crane to hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert leveling pad under machine bed

- Using of forklift to unload machine accessories

- Uncrate, shift and position into factory premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175