# METHOD OF STATEMENT

Date: 17th Dec 2025

Description of Work

To unload 3 cases of 1 unit machine from low-bed/lorry chassis, shift and position into production without uncrating

Machine Spec

Global Advantage Scan+ 20.33.18 CMM

Case 1: 460 x 280 x 110 cm

N/G Wt: 14, 500 kg / 17, 000 kg

Case 2: 376 x 140 x 305 cm

N/G Wt: 650 kg / 1, 450 kg

Case 3: 474 x 117 x 96 cm

N/G Wt: 620 kg / 1, 448 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/12/25

Time of Operation

10:00 am

Location of Operation

#### 1 Seletar Aerospace Crescent

Rolls-Royce Singapore Pte Ltd

Equipment Use

1 x 110ton lorry loader

633 lorry loader

Rollers

Wire sling

Webbing sling

Shackle

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Setting up of crane with outrigger fully extended and resting on metal plate as base plate

- Riggers to insert sling to crane hook

- Secured sling to rigging point of case

- Signalman to give signal to crane operator to hoist down case from low-bed to the ground

- Insert rollers under case bed

- Shift and transport into factory premise at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175