# METHOD OF STATEMENT

Date: 03rd Apr 2023

Description of work

To unload of 1 unit machine from 1 x 40ft HQ container, shift and transport into factory premise at ground floor

Machine Spec

1 unit Mazak M5N CNC Lathe

Dim: 600 x 228 x 225 cm

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/04/23

Time of Operation

09:45 am

Location of Operation

#### 10 Gul Circle

OMS Oilfield Services Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Pull out machine from container onto lorry chassis

- Hoist down from lorry chassis and load onto ground

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at temporary location

- Once job complete, team leader will instruct team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175