# METHOD OF STATEMENT

Date: 28th Oct 2023

Description of work

Transport lorry into inside factory, using customer overhead crane to hoist up cases onto lorry chassis, transfer to new plant, unload, uncrate, shift and position into premise

Machine Spec

1 unit CMM LH 1210-Next Generation

Case 1: 470 x 220 x 137 cm / Wt: 12, 000 kg

Case 2: 228 x 100 x 100 cm / Wt: 530 kg

Case 3: 218 x 195 x 110 cm / Wt: 700 kg

Case 4: 135 x 110 x 120 cm / Wt: 140 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/12/23

Time of Operation

09:45 am

Location of Operation

#### 10 Gul Circle

OMS Oilfield Services Pte Ltd

Equipment Use

1-2 x lorry loader

Customer 20ton overhead crane

Wire/webbing sling

Shackle

Canvas/metal sheet (to lay out for floor protection)

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport of lifting machinery into inside factory premise

- Insert sling under machine case

- Using customer overhead crane to hoist up and load cases onto from lorry chassis

- Transfer case to new plant

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Rigger will attached sling to crane hook and secured to machine rigging point

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175