# METHOD OF STATEMENT

Date: 11th Sep 2023

Description of work

To shift out 1 unit machine and accessories from production area to loading bay, hoist up onto low loader and transport to scrapyard for disposal

Machine Spec

1 x Toshiba BMC-80E

Dim: 650 x 360 x 320 cm

Wt: 18, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/09/23 (Afternoon)   -   shift out accessories

13/09/23 (Morning)   -   shift out machine

Time of Operation

09:45 am

Location of Operation

#### 10 Riverside Rd

Okamoto (S) Co. Pte Ltd

Equipment Use

1 x 50 ton mobile crane

633 lorry loader

1 x low loader

1 x forklift

Hydraulic jack

Rollers

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated loading area

- Shift out machine from production to loading area

- Rigger will attached wire sling to crane hook and secure to rigging point of machine

- Hoist up machine

- Reverse low loader to near crane

- Unload machine onto low loader chassis

- Release all sling from rigging point of machine

- Secure machine with chain belt

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175