# METHOD OF STATEMENT

Date: 06th Mar 2025

Description of Work

To unload 2 cases of 1 unit machine from low loader, uncrate case, shift and position into production area at ground floor

Machine Spec

1 unit YV100ATC+G machine

Dim: 424 x 345 x 399 cm / Wt: 13, 460 kg

Dim: 455 x 261 x 282 cm / Wt: 11, 580 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

14/03/25

Time of Operation

10:00 am

Location of Operation

#### 100 Seletar Aerospace View

Vector Aerospace Asia Pte Ltd

Equipment Use

110ton lorry crane

633 lorry crane

1 x low loader

1 x forklift

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Hydraulic jack

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Insert sling under case

- Lorry crane to hoist down case from low loader to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Engineer to install machine at outside factory

- After install, shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175