# METHOD OF STATEMENT

Date: 17th Aug 2022

Description of work

To unload 7 packages of 1 unit machine from 2 x 40ft HQ container, shift and transport into factory premise at ground floor

Machine Spec

1 unit Ultrahone Horizontal Honing

1 skid – Horning machine base / Wt: 8, 200 kg

1 skid – 55 GAL-Horning Oil-Lubricant-Non Hazmat / Wt: 861 kg

1 skid – Horning machine work base / Wt: 6, 805 kg

1 skid – 55 GAL-Honing Oil-Lubricant-Non Hazmat / Wt: 430 kg

1 skid – Horning stone / Wt: 135 kg

1 bundle – Filter paper / Wt: 56 kg

1 crate – Tools & accessories / Wt: 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/08/22

Time of Operation

09:45 am

Location of Operation

#### 11 Tuas Ave 6

Drilmaco Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling/webbing sling

Woodblock

Crowbar/hand jack

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back to back with container)

- Pull out skid from container onto lorry chassis

- Insert sling to crane hook and secure onto machine rigging point

- Hoist down machine from lorry chassis to the ground

- Loosen all nuts from machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175