# METHOD OF STATEMENT

Date: 26th Aug 2024

Description of Work

To hoist down case from 1 x 20ft FR container, uncrate, shift and position into production area at ground floor

Machine Spec

3 cases of 1 Vertical Machining Center / YCM-NXV-1680B

Case 1: 476 x 374 x 377 cm

N/G Wt: 18, 820 kg / 16, 82 kg

Case 2: 470 x 146 x 200 cm

N/G Wt: 1, 010 kg / 800 kg

Case 3: 126 x 111 x 75 cm

N/G Wt: 640 / 570 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/08/24

Time of Operation

09:45 am

Location of Operation

#### 110 Tuas South Ave 3 #01-06

#### RTHT Technologies Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x 633 lorry loader

1 x scissor lift

Rollers/pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

PPE Required

Safety helmet

Safety shoe

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a) Lifting Supervisor

b) Mobile Crane Operator

c) Lorry Loader Operator

d) Signalman

e) Signalman

f) Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Set up of crane with outrigger fully extended and resting on metal plate/timber mats as base plate

- To hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175