# METHOD OF STATEMENT

Date:20th June 24

Description of work

To unload  machine from 1 x 20ft FR container, uncrate, shift and position into factory premise at ground floor

Machine Spec

Dim: 3200 x 4300 x 3200mm / Wt:11.500kgs

Dim: 3010 x 4110 x 3070mm / Wt 9.500kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

25/06/24

Time of Operation

09:45am

Location of Operation

#### 12 Sixth Lok Yang Rd

Kim Ann Egineering Pte Ltd

Equipment Use

1 x 633 crane

Factory overhead crane

Wire sling/webbing sling

Rollers

Shackle

Hydraulic jack

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane near FR container

- Setting up of crane with outriggers fully extended and resting on metal plate as base plate

- To hoist down case from container to the ground

- To uncrate machine case and put aside

- Rigger will attached required lifting gears to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Using customer overhead crane to hoist up machine and transfer to near door entrance

- Insert rollers under machine bed

- Shift and transport machine into factory production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

Best Regards

Kevin / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175