# METHOD OF STATEMENT

Date: 13th Jan 2023

Description of work

To unload 5 cases of 1 unit machine from 1 x 40ft HQ container, uncrate, shift and transport machine into factory premise at ground floor

Machine Spec

1 unit Mazak VCP-400L Vertical Machining Center

Crate 1: 3100 x 1700 x 2520 mm / Wt: 5, 020 kg

Crate 2: 3000 x 1500 x 930 mm / Wt: 450 kg

Crate 3: 3600 x 900 x 1300 mm / Wt: 530 kg

Crate 4: 1800 x 1100 x 1100 mm / Wt: 220 kg

Crate 5: 2200 x 1000 x 1700 mm / Wt: 450 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

30/01/23

Time of Operation

Afternoon schedule

Location of Operation

#### 12 Tukang Innovation Drive #01-01

Dormakaba Production GMBH & CO.KG

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Woodblock

Crowbar/hand jack

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back to back with container)

- Pull out case from container onto lorry chassis

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Attached sling to crane hook and secured to machine rigging point

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175