# METHOD OF STATEMENT

Date: 24th July 2025

Description of Work

To unload 30 cases of 6 units machine and accessories from Malaysia low-bed and lorry, shift and position into production area at ground floor

Machine Spec

Refer to packing list attach

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est:

31/07/25 – 01/08/25 (to unload 2 units CNC Machining Center)

05/08/25 – 06/08/25 (to unload 2 units CNC Machining Centre)

07/08/25 – 08/08/25 (to unload 2 units CNC Machining Centre)

Time of Operation

10:00 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

70ton mobile crane

633 lorry loader

1 x short tail low bed

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated unloading area for the lifting work

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Reverse low-bed to near crane

- Insert sling below machine case

- Hoist down case from low-bed to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Load up machine onto short tail low bed and transfer to behind door entrance

- Hoist down machine and lower down to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175