# METHOD OF STATEMENT

Date: 05th Aug 2022

Description of Work

To unload machine and equipment from lorry chassis, shift and position into tenant premise at ground floor

Machine Spec

1 unit Hydraulic Press

Dim: 125 x 90 x 185 cm / Wt: 300 kg

A Frame

Hoist structure

Scissor lift trolley

Stainless steel trolley

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/08/22

Time of Operation

13:00 pm

Location of Operation

#### JTC Space @ 164 Gul Circle #01-02

Evoqua Water Technologies Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck / rollers

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Insert sling to crane hook and secure onto machine rigging point

- Lorry loader to hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine base

- Shift and position into tenant premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175