# METHOD OF STATEMENT

Date: 15th July 2024

Description of work

To hoist down accessories from lorry chassis, shift and position into factory premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/07/24

Time of Operation

Est: 11:30 am

Location of Operation

#### 16E Tuas Ave 1 #01-61

Hunting Energy Services Pte Ltd

Equipment Use

633 lorry loader

1 x forklift

Rollers

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Setting up crane on site with outrigger fully extended and resting on metal plate/timber mats as base plate

- Riggers to attached sling to crane hook

- Secure sling to rigging point of accessories

- Hoist up from lorry chassis

- Swing crane and lower down to the ground

- Insert pallet truck

- Transport into factory premise

- Position at the designated location

- Using forklift to unload accessories from lorry

- Shift and transport into factory premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175