# METHOD OF STATEMENT

Date: 08th Jan 2024

Description of work

To shift out machine from production area to loading bay, load up onto lorry chassis, transport out to next building, hoist down, shift and position into factory premise

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/01/24

Time of Operation

09:40 am

Location of Operation

#### 17 Kian Teck Crescent

Perfect Team Engineering Pte Ltd

Equipment Use

2 x 633 lorry loader

Rollers

Hydraulic jack

Woodblock

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading/unloading bay and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up and insert rollers under machine bed

- To shift out machine from production area to loading bay

- Reverse lorry to loading bay

- Level up lorry chassis with loading bay

- Load up machine onto lorry chassis

- Transport machine out from loading bay

- Deploy lorry at unloading area

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist down machine to the ground

- Insert rollers under machine bed

- Shift and transport into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175