# METHOD OF STATEMENT

Date: 26th May 2022

Description of work

To shift out machines and racking from production room to loading area, hoist up and load onto wooden base for packing, after packing load onto low loader & 1 x 20ft GP container

Spec

3 unit Italcontrol Test machine

3 unit machine base tray

4 unit racking with materials

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

31/05/22

Time of Operation

09:45 am

Location of Operation

#### 2 Gul Circle

#### Cameroon

Equipment Use

1 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Packing materials

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers to machine bed

- Shift out machine from existing location

- Transport machine to designated hoisting/packing area

- Attached lifting gears to crane hook and secure to hoisting point of machine to be lifted

- Hoist up machine and load onto wooden base

- Pack to form a case

- After packing, hoist up case and load case onto container

- Secured case with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175