# METHOD OF STATEMENT

Date: 08th May 2024

Description of Work

To unload 11 steel case of 2 unit machine from 3 x 40ft FR, 1 x 40ft GP & 1 x 20ft FR container, uncrate, shift and position into production area at ground floor

Machine Spec

Refer to attached packing list

5 cases of 1 unit HN8800E

6 cases of 1 unit VRi600N

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/05/24 – to unload VRi600N

17/05/24 – to unload HN8800N

Time of Operation

10:00 am

Location of Operation

#### 2 Seletar Aerospace Link

Avimac Pte Ltd

Equipment Use

1 x 110 ton lorry crane

2 x 633 lorry crane

1 x forklift

1 x 30ft low-bed

Rollers/pallet truck

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Rigger to insert sling below machine case

- Lorry crane to hoist down case from container to the ground

- Uncrate case and put aside

- Unscrew all nuts from machine bed

- Insert sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Load up machine onto low-bed

- Transfer machine to door entrance

- Hoist down machine from low-bed

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175