# METHOD OF STATEMENT

Date: 12th Apr 2023

Description of Work

To unload 3 cases of 1 unit machine from 1 x 20ft FR and 1 x 20ft GP container, uncrate case, shift and position into production area at ground floor

Machine Spec

1 unit NSV106A CNC

Case 1: 3.57 x 2.36 x 3.10 m

Wt: 8, 370 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/04/23

Time of Operation

09:45 am

Location of Operation

#### 2 Woodlands Sector 1 #01-29

ST Engineering Aerospace Precision Products Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Manual/hydraulic jack

Metal plate/canvas (for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader near container for the lifting work

- Insert sling below machine case

- Hoist down case from FR container to the ground

- Uncrate case and put aside

- Insert sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport into factory premise at ground floor

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175