# METHOD OF STATEMENT

Date: 06th Dec 2022

Description of work

Using lorry loader to hoist up and lay down machine, load onto low bed and deliver to new factory

Machine Spec

1 unit HY 201 Hydraulic Press machine

Wt: 15, 000 kg

1 unit Komatsu 200 Single Press

Wt: 20, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/12/22 (HY 201 Hydraulic Press)

28/12/22 (Komatsu 200 Single Press)

Time of Operation

09:45 am

Location of Operation

#### Shimano (S) Pte Ltd

Move out: 20 Benoi Sector

Move in: 11 Bulim Walk

Equipment Use

1 x 100 ton lorry loader

1 x 633 lorry loader

1 x low bed

Heavy duty rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry loader into inside factory

- Deploy 2 x lorry loader to near machine

- Attached required lifting gears to crane hook and secured to lifting point of machine to be lifted

- 100ton lorry loader to hoist up machine

- Use 50ton lorry loader for assist to lay down machine to horizontal position

- Hoist up machine and load onto low bed

- Secured machine with strapping belt/chain prior to being transport out from premise

- Deliver machine to new factory

- Unload machine form low bed

- 2 x lorry loader to upright machine

- Insert rollers under machine bed

- Shift and transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175