# METHOD OF STATEMENT

Date: 30th Dec 2022

Description of work

To hoist up 2 unit machine to 2nd floor through swing gate opening by using platform and lorry crane

Machine Spec

2 unit Cutex 180A

Dim: 250 x 170 x 199 cm

Wt: 3, 600 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/01/23 (Friday)

Time of Operation

9:45 am

Location of Operation

#### Blk 20 Woodlands Link #02-37

#### Yi Xin Precision Engineering Pte Ltd

Equipment Use

2-3 x lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Tied back platform to a competent point and secured using 1 x lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert pallet truck/rollers under machine base

- Shift out machine from platform

- Position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175