# METHOD OF STATEMENT

Date: 14th July 2025

Description of Work

To hoist up 6 units machine to 2nd floor through shutter door opening by using lorry loader

Machine Spec

2 unit 2m Lathe

1 unit 1.5m Lathe

1 unit 1m Lathe

2 units 5HP Milling

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

29/07/25

Time of Operation

Est:

13:00 pm

Location of Operation

#### 21 Buroh Street

HMB Precision Engineering & Services

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety shoe

Safey helmet

Safety harness

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Signalman

d. Riggerman

e. Skill Mover

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry loader to hoist up machine to 2nd floor through shutter door opening

- Lower down machine to the ground

- Insert rollers/pallet truck under machine base

- Shift machine into factory premise

- Position machine at the designated location

- Unload machine and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175