# METHOD OF STATEMENT

Date:

Description of work

To unload 1 unit machine and accessories from 4 x 40ft OT container at outside factory premise, assist engineer to install machine parts, shift and position into customer premise at ground floor using rollers

Machine Spec

Danobat Grinding / Danuni 4100

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

09:30 am

Location of Operation

#### 21 Loyang Crescent

Safran Landing Systems Services Singapore Pte Ltd

Equipment Use

1 x 70 ton mobile crane

Lorry loader

Heavy duty rollers

Pallet truck

8/10 ton wire sling

10 ton webbing sling

Shackle

Canvas/metal sheet (to lay out for epoxy floor protection)

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                                                                                                                                                                           Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# P.P.E Required

# Safety helmet

# Safety shoe

# Glove

# JOB SCOPE

- Deployment of mobile/lorry crane to side location

- Setting up of mobile crane at designated location for the lifting

- Open up container canvas

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up machine from container and load onto ground

- Assist engineer to install machine parts

- After installing, insert rollers under machine bed and secured with bolt and nuts

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling plate under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175