# METHOD OF STATEMENT

Date: 26th Apr 2021

Description of work

To hoist up 4 unit machine, load onto low bed/lorry chassis and deliver to Eric Wong warehouse

Machine Spec

2 unit China Press Single Crank 63 / Wt: 6, 000 kg each

2 unit China Press Single Crank 160 / Wt: 14, 000 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 28-30/04/21

Time of Operation

09:45 am

Location of Operation

#### 21 Tanjung Penjuru

Singapore Highpolymer Chemical Products

Equipment Use

2 x 633 lorry loader

1 x low bed

Heavy duty rollers

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry loader into inside factory premise

- Attach required lifting gears to crane hook and secure onto hoisting point of machine to be lifted

- 1 x lorry loader to hoist up machine

- Using another lorry loader for assist to lay down machine to horizontal position

- Hoist up machine and load onto low bed/lorry chassis

- Secured machine with strapping belt prior to being transport out from premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175