# METHOD OF STATEMENT

Date: 17th Oct 2022

Description of work

To unload 4 cases of 2 unit from 1 x 40ft HQ container using lorry loader, uncrate, shift and position to designated location using rollers

Machine Spec

EMCOTURN E65 SM
Case 1: 367 x 227 x 253 cm / G/N Wt: 6, 440 kg / 5, 400 kg

Case 2: 367 x 227 x 253 cm / G/N Wt: 6, 440 kg / 5, 400 kg

Case 3: 240 x 94 x 175 cm / G/N Wt: 600 kg / 400 kg

Case 4: 240 x 94 x 175 cm / G/N Wt: 600 kg / 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/09/22

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

ST Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 10ton forklift

1 set wire/webbing sling

Shackle

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back to back with container)

- Crane to pull out case from container and load onto lorry chassis

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Attached required lifting gears onto crane hook and secured to rigging point of machine to be lifted

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 6282417