# METHOD OF STATEMENT

# Date: 21st Nov 2024

Description of work

To unload 2 unit machine from 2 x 40ft HQ container, uncrate, shift and position into factory premise at ground floor

Machine Spec

2 unit Goodway GS-3600L

Dim: 442 x 220 x 226 cm each

Wt: 9, 000 kg each

1 unit robot arm

Wt: 1, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

28/11/24

Time of Operation

09:45 am

Location of Operation

#### 249 Jalan Boon Lay

ST Engineering Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x 10ton forklift

1 set wire/webbing sling

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift driver will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety vest)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Reverse lorry back to back with container

- Setting up of lorry crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Crane to pull out case from container and load onto lorry chassis

- Using of 10ton forklift to fork down case to the ground

- Uncrate case and put aside

- Forklift to fork up and detach machine from base

- Transport and unload machine at door entrance

- Using rollers to transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175