# METHOD OF STATEMENT

Date: 08th Nov 2024

Description of work

To unload 2 unit machine and 1 unit robot arm from 2 x 40ft HQ container using lorry loader, shift and position to the designated location at ground floor using rollers

Machine Spec

2 unit Goodway GS-3600L

Dim: 442 x 220 x 226 cm each

Wt: 8, 000 kg each

1 unit robot arm

Wt: 1, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/11/24 (Tuesday)

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

ST Engineering

Equipment Use

1-2 x 633 lorry loader

1 set wire/webbing sling

Shackle

Rollers

Pallet truck

Hand jack

Woodblock

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Reverse lorry back to back with container

- Crane to pull out case from container and load onto lorry chassis

- Riggers to attached required lifting gears onto crane hook

- Secured to rigging point of machine to be lifted

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 6282417