# METHOD OF STATEMENT

Date: 28th Feb 2025

Description of work

1) To hoist down 2 skid of 1 unit machine from lorry chassis, shift and position into factory premise at ground floor

2) To shift out 1 unit Bandsaw, 1 unit 2m roller table from production and load up onto lorry chassis

Machine Spec

Move in

Bandsaw/Cosen 05SH-7662

Dim: 376 x 158 x 229 cm / Wt: 3, 462 kg

Dim: 97 x 216 x 61 cm / Wt: 215 kg

Move out

1 unit Bandsaw/Daito Seiki

Wt: 2, 600 kg

1 unit 2m roller table

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

/03/25

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

ST Engineering Advanced Material

Equipment Use

1 x lorry loader

Wire/webbing sling

Shackle

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Rigger to insert required sling onto crane hook

- Secured sling to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Shift machine into production at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- To shift out 1 unit old machine from production to loading bay

- Hoist up onto lorry chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 6282417