# METHOD OF STATEMENT

Date: 26th Oct 2023

Description of work

To unload machineries from trailer/lorry chassis using lorry crane and forklift, shift and position into factory premise at ground floor

Machine Specification

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

01-03/11/23

Time of Operation

Estimate:

12:00 pm

Location of Operation

249 Jalan Boon Lay

ST Engineering Pte Ltd

Equipment Use

Lorry loader

Trailer

1 x 10ton forklift

Wire/webbing sling

Shackle

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert required sling onto crane hook

- Secured sling to rigging point of machine to be lifted

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Using of forklift to fork down machine from lorry chassis

- Transport and unload machine near door entrance

- Insert rollers/pallet truck under machine bed

- Shift and transport into production at ground floor

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 6282417