# METHOD OF STATEMENT

Date: 08th Dec 2025

Description of work

To hoist down 2 units machine from lorry chassis, shift and position into factory premise at ground floor

Machine Spec

1 unit CNC Wire Cut AR2 Pro 450

Dim: 190 x 200 x 210 cm

Wt: 1, 800 kg

1 unit CNC Wire Cut AF50

Dim: 238 x 161 x 240 cm

Wt: 2, 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/12/25

Time of Operation

09:00 am

Location of Operation

249 Jalan Boon Lay

STEA Precision Pte Ltd

Equipment Use

1 x lorry loader

Wire/webbing sling

Shackle

Pallet truck

Crowbar

Woodblock

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Rigger will insert required sling onto crane hook and secure to rigging point of machine

- Hoist up machine from lorry chassis

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift machine into production area at ground floor

- Position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 6282417