# METHOD OF STATEMENT FOR 4th SHIPMENT (SHANGHAI SHIPMENT)

Date: 27th Nov 2023

Description of work

To unload 2 unit machine from 1 x cometto trailer and 1 x super heavy duty low-bed, uncrate, shift and position to temporary storage location arranged by customer

Machine Spec

Hot Forging Press 1250T

Dim: 9950 x 2450 x 2260 mm

Wt: 66, 730 kg

Hot Drawing Press 400T

Dim: 11370 x 2000 x 2000 mm

Wt: 39, 530 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate schedule:

02/12/23

Time of Operation

09:45 am

Location of Operation

#### 249 Jalan Boon Lay

ST Engineering Pte Ltd

Equipment Use

1 x 250ton mobile crane

Lorry loader

1 x cometto

1 x super heavy low-bed

Electric rollers

Hydraulic jack

Wire sling

Webbing sling

Shackle

1 x 4.5ton forklift

1 x scissor lift

Metal sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety vest)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport 250ton mobile crane into customer factory premise

- Deploy mobile crane at the designated unloading area

- Reverse cometto to near mobile crane

- Riggers to attached sling to crane hook and secure to rigging point of case

- Mobile crane to hoist up case from container

- Unload case to the ground

- Uncrate side cover of case and put aside

- Insert electrical rollers onto base and secured with bolts and nuts

- Transport to temporary storage area

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175