# METHOD OF STATEMENT

07th May 2024

Description of work

To unload 8 cases of 1 unit machine and accessories from 1 x 40ft FR, 1 x 20ft GP and 1 x 40ft GP container, uncrate, shift and position into factory premise

Machine Spec

VARIAXIS I-300AWC

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

20/05/24

Time of Operation

09:45 am

Location of Operation

#### 249 Jalan Boon Lay

ST Engineering Pte Ltd

Equipment Use

Lorry loader

Electric rollers/pallet truck

Hydraulic/hand jack

Wire sling

Webbing sling

1 x 16ton forklift

1 x 3ton forklift

1 x scissor lift

Metal sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift driver will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety vest)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

For 20ft & 40ft GP container

- Transport lorry crane into customer factory premise

- Reverse lorry crane back to back with container

- Using crane to pull out cases and load onto lorry chassis

- Using forklift to fork down cases to the ground

- Uncrate case and put aside

- Using of lorry crane or forklift to lift up and detach accessories from base

- Using rollers/pallet truck to transfer into factory premise

- Position at the designated location

For 1 x 40ft FR container

- Using of 16ton forklift to fork up case from container

- Unload case to the ground

- Uncrate case and put aside

- Using forklift to fork up and detach machine from base

- Transport to door entrance

- Lower down machine to near ground level

- Inset rollers under machine bed

- Transport machine into factory premise

- Position at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175