# METHOD OF STATEMENT

Date: 08th Nov 2023

Description of work

To unload 48 case of machine and accessories from 1 x 40ft FR, 4 x 40ft OT and 3 x 40ft HQ container, uncrate, shift and position to temporary storage location arranged by customer

Machine Spec

Straight Side Hydraulic Press HDIS-630

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate schedule:

14-17/11/23

Time of Operation

09:45 am

Location of Operation

#### 249 Jalan Boon Lay

ST Engineering Pte Ltd

Equipment Use

1 x 100ton lorry crane

Lorry loader

Electric rollers/pallet truck

Hydraulic jack

Wire sling

Webbing sling

Shackle

1 x 10ton forklift

1 x 16ton forklift

1 x boom/scissor lift

Metal sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane to site location

- Setting up of lorry crane at designated unloading area

- Riggers to insert sling under machine case

- Hoist up case to check on balancing

- When well balancing, hoist down case from container

- Unload case to the ground

- Uncrate case and put aside

- Attached sling to crane hook and secure to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine slowly to near ground level

- Insert hydraulic rollers to machine base and secured with bolts and nuts

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175