# METHOD OF STATEMENT

Date: 09th Nov 2021

Description of work

To unload 1 unit machine from lorry chassis, shift and position into factory premise at ground floor

Machine Spec

Hurco VMX50 machine
Dim: 3280 x 2282 x 2519 mm

Wt: 7, 850 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

23/11/2021

Time of Operation

09:45 am

Location of Operation

249 Jalan Boon Lay

ST Engineering Land Systems

Equipment Use

1 x 633 lorry loader

1 set wire sling

Shackle

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Attached required lifting gears onto crane hook and secured to rigging point of machine to be lifted

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck onto machine base

- Shift machine into production area at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 6282417