# METHOD OF STATEMENT

Date: 16th July 2024

Description of work

1) To shift out 1 unit Oven and 1 unit Printing machine from 5th floor to loading area via cargo lift, hoist and load up onto lorry chassis for disposal

2) To relocate 7 unit Oven within production at 5th floor

Risk Assessment

A copy of Risk Assessment will be attach

Schedule of Operation

29/07/24

Time of Operation

09:30 am

Location of Operation

#### 25 Pandan Crescent

N & U

Equipment Use

1 x lorry loader

Rollers/pallet truck

Crowbar

Wire / webbing sling

Obstacles

No obstacle on crane parking area and route of moving operation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

# (1)

- Lift up machine to 3” high using crowbar

- Insert pallet truck under machine bed

- Shift out machine from production to hoisting bay via cargo lift

- Rigger to attached required sling to crane hook

- Secured sling to rigging point of load to be lifted

- Lorry loader to hoist up and load machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver to scrapyard for disposal

(2)

- Lift up machine to 3” high using crowbar

- Insert pallet truck under machine bed

- Shift out oven from existing location

- Transfer to new location within production

- Position at the new designated location

- Unload from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175