# METHOD OF STATEMENT

Date: 10th May 2024

Description of work

To unload 1 unit machine from container, uncrate, shift and transport into QA room at ground floor

Machine Spec

1 unit 15.30.12 CMM machine

Dim: 450 x 215 x 230 cm

Wt: 9, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

14/05/24 (Tue)

Time of Operation

10:00 am

Location of Operation

#### 26 Changi North Crescent

Smartt Precision MFG

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Hydraulic/hand jack

Woodblock

Canvas/metal plate (for lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down cases from forwarder lorry to the ground

- Uncrate case

- Hoist up and detach machine from base

- Load up machine onto lorry chassis

- Secure machine with strapping belt

- Reverse lorry to loading bay

- Jack up and insert rollers under machine bed

- Shift out machine from lorry chassis

- Transport machine into QA room 80m away

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175