# METHOD OF STATEMENT

Date: 02nd Aug 2023

Description of work

To unload 2 unit machine and accessories from low loaders, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit Mazak Integrex E650

1 unit Mazak Integrex E500

Refer to packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 10-31/08/23

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte

Equipment Use

1 x 250 ton mobile crane

Lorry loader

Hydraulic rollers

15ton x 12m webbing

15ton wire sling

17-25 ton shackle

2 x 3m 25ton spreader bar

1 x 3 ton battery forklift

1 x boom/scissor lift

Metal sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated unloading area

- Reverse low loader to near crane

- Insert sling under machine case

- Use of 2 x spreader bar to prevent damage to case when hoisting

- Hoist up case to check on balancing

- When well balancing, hoist down case from low loader

- Unload case to the ground

- Uncrate case and put aside

- Attached sling to crane hook and secure to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine slowly to near ground level

- Insert hydraulic rollers to machine base and secured with bolts and nuts

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175