# METHOD OF STATEMENT

Date: 25th Sep 2024

Description of Work

To hoist down 8 cases of 1 unit machine from 2 x 40ft FR and 2 x 40ft GP container, uncrate, shift and position into production area at ground floor

Machine Spec

8 cases of 1 unit Mazak FJV60/80

Case 1: 609 x 368 x 370 cm / Wt: 22, 910 kg

Case 2: 494 x 228 x 252 cm / Wt: 4, 410 kg

Case 3: 630 x 311 x 254 cm / Wt: 7, 140 kg

Case 4: 580 x 228 x 222 cm / Wt: 870 kg

Case 5: 353 x 179 x 109 cm / Wt: 620 kg

Case 6: 270 x 175 x 107 cm / Wt: 4, 480 kg

Case 7: 350 x 218 x 74 cm / Wt: 360 kg

Case 8: 365 x 120 x 204 cm / Wt: 700 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

/24

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

#### Panasonic Factory Solution Asia Pacific

Equipment Use

1 x 50 ton mobile crane

2 x 633 lorry loader

1 x scissor lift

Electric roller/rollers

Pallet truck

Wire/webbing sling

Shackle

Hydraulic jack

Woodblock

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

PPE Required

Safety helmet

Safety shoe

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a) Lifting Supervisor

b) Mobile Crane Operator

c) Lorry Loader Operator

d) Riggerman

e) Signalman

f) Skill Mover

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Set up of crane with outrigger fully extended and resting on metal plate/timber mats as base plate

- To hoist down case from FR container to the ground

- Uncrate case and put aside

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Signalman to give signal to crane operator to hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175