# METHOD OF STATEMENT

Date: 07th Jan 2025

Description of Work

To hoist up 1 unit machine to 2nd floor through shutter door opening by using lorry loader

Machine Spec

Mazak QT Compact 100

Dim: 260 x 170 x 175 cm

Wt: 4, 600 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/01/25

Time of Operation

12:00 am

Location of Operation

#### 2A Changi North St 2

ETLA Limited

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Shackle

Canvas/PVC sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety net, red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry loader to hoist up machine to 2nd floor shutter door opening

- Lower down machine to the ground

- Insert rollers/pallet truck under machine base

- Shift machine into factory premise

- Position machine at the designated location

- Unload machine and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175