# METHOD OF STATEMENT

Date: 23rd July 2025

Description of work

To hoist down machine from 3rd floor open space balcony to the ground and load up onto lorry chassis

Machine Spec

2 units Carton Boxes Maker machine

Dim: 230 x 175 x 220 cm / Wt: 2, 800 kg

1 unit Conveyor

Dim: 360 x 130 x 240 cm / Wt: 2, 000 kg

1 unit Controller

Dim: 110 x 50 x 180 cm / Wt: 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est. 25/08/25 - 31/08/25

Time of Operation

09:30 am

Location of Operation

#### 3 Cleantech Loop #01-01 (3rd floor open space balcony)

Advanced Remanufacturing & Technology Centre

Equipment Use

1 x 50ton mobile crane

2 x lorry

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Loader Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Deployment of mobile crane at designated hoisting location for the lifting work

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- To extract crane boom to 3rd floor open space balcony

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Mobile crane to hoist up machine 1ft above ground to check on balancing

- When well balance, hoist up machine from 3rd floor balcony

- Swing crane and lower down machine to the ground

- Load up machine onto lorry chassis

- Secured with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175