# METHOD OF STATEMENT

Date: 19th July 2022

Description of work

To hoist down 41 units Compressor from roof top to ground floor and load onto customer arranged forwarder lorry

Compressor Spec

Dim: 80 x 80 x 80 cm

Wt: 100 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/07/22 (Mon)

Time of Operation

09:30 am

Location of Operation

#### 3 Toh Guan Rd East

Knight Auto Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton mobile crane

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated location for the hoisting work

- Crane operator to extract the crane boom to building rooftop

- Insert sling to crane hook and secure onto rigging point of compressor

- Hoist up 1ft above ground to check on balancing

- When well balance, hoist down compressor from roof top to the ground

- Load up compressor onto lorry chassis

- Release all sling form compressor rigging point

- Secured compressor with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175