# METHOD OF STATEMENT

Date: 02nd Mar 2021

Description of work

To unload 2 cases of 2 unit machine from 1 x 40ft HQ container, shift and transport to temporary storage location at ground floor

Machine Spec

2 unit Roku Roku Milling machine

Case dim: 3090 x 2250 x 2480 mm

Wt: 3, 600 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/03/2021

Time of Operation

09:45 am

Location of Operation

#### 3 Toh Guan Rd East

Knight Auto Precision Engineering Pte Ltd

Equipment Use

2 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area (back to back with container)

- Pull out machine case from container onto lorry chassis

- Hoist down machine case from lorry chassis and load onto ground

- Insert pallet truck under machine case

- Shift and transport case to temporary storage location at ground floor

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175