# METHOD OF STATEMENT

Date: 01st Nov 2024

Description of work

To shift out 1 unit press machine and accessories from production to loading area, hoist and lay down machine, load onto 1 x 40ft FR and 1 x 40ft HQ container

Machine Spec

1 unit GSL-300S Patec Press

Wt: 34, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

05/11/24 – 16/11/24

Time of Operation

09:45 am

Location of Operation

#### 33 Gul Lane

Bauman Spring Company (S) Pte Ltd

Equipment Use

1 x 100ton mobile crane

1 x 633 lorry crane

1 x low bed

1 x forklift

Heavy duty rollers

Wire sling/shackle

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

P.P.E Required

Safety helmet

Safety shoe

Safety Aspect

Assignment of traffic controller to movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Mobile Crane Operator

c. Lorry Crane Operator

d. Signalman

e. Riggerman

f. Skill Mover

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production area

- Use of forklift to assist in pulling/pushing machine to loading bay

- Rigger to attached required lifting gears to crane hook

- Secured sling to rigging point of machine

- Mobile crane to hoist up machine 1ft above ground

- Use 55ton lorry loader to assist laying down machine to horizontal position

- Hoist up machine and load onto FR container

- Secured machine with strapping belt/chain sling and chocking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175