# METHOD OF STATEMENT

Date: 22nd Jan 2024

Description of work

To shift out 3 machine from production room to loading area, hoist up and load machine onto low bed and deliver to new location

Machine Spec

1 unit LVD PPEC-4320/45 Press Brake

1 unit LVD MVCB 31/10 Shearing

1 unit Rolling machine

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

24/01/24

Time of Operation

09:45 am

Location of Operation

#### 34 Joo Koon Circle

Zenith Engineering Pte Ltd

Equipment Use

1 x 110ton lorry loader

1-2 x 633 lorry loader

1 x low loader

1 x forklift

Heavy duty rollers

Wire sling/shackle

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production area

- Use of forklift to assist in pulling/pushing machine to loading bay

- Rigger to attached required lifting gears to crane hook

- Secured sling to rigging point of machine to be lifted

- Hoist up machine

- Load up onto low bed

- Secured machine with strapping belt/chain

- Transport out and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175