# METHOD OF STATEMENT

Date: 09th Dec 2025

Description of work

To hoist down 1 unit machine from 2nd floor open space balcony to the ground and load up onto lorry chassis

Machine Spec

1 unit Mori NLX 2500/700 machine

Dim: 330 x 190 x 190 cm

Wt: 5, 400 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/12/25

Time of Operation

09:30 am

Location of Operation

#### 5 Cleantech Loop, Cleantech 2B (2nd floor open space balcony)

ARTC

Equipment Use

1 x 633 lorry loader

Roller/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

Lifting machine and lifting gear with M.O.M certificate

Lifting crew shall be composed to the following but not limited by other trained personnel in respect to number of crew needed for the operation:

a. Lifting Supervisor

b. Lorry Loader Operator

c. Signalman

d. Riggerman

e. Skill Mover

# JOB SCOPE

- Deployment of lorry crane at designated hoisting location for the lifting work

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Barricade off work area with safety cones, red/white tape and lifting in operation signage

- To extract crane boom to 2nd floor open space balcony

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Lorry crane to hoist up machine 1ft above ground to check on balancing

- When well balance, hoist up machine from 2nd floor balcony

- Swing crane and lower down machine to the ground

- Load up machine onto lorry chassis

- Secured with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175