# METHOD OF STATEMENT

Date: 29th Apr 2024

Description of work

1) To shift out 1 unit Pan Blast machine from production to loading area, hoist up onto lorry chassis, transfer to another building within 200m away, unload, shift and position into production

2) Using forklift to remove Dust Collector bolting and machine leg, shift out and load onto lorry chassis, transfer to another building, unload and position outside factory

Machine Spec

1 unit Pan Blast machine

Dim: 600 x 250 x 320 cm / Wt: 3, 500 kg

1 unit Dust Collector

Dim: 240 x 150 x 490 cm / Wt: 1, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01/06/24

Time of Operation

10:00 am

Location of Operation

#### 501 Airport Rd

ST Aerospace Engines Pte Ltd

Equipment Use

1 x lorry loader

Rollers/pallet truck

Crowbar

Woodblock

1 x forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

# (1)

- Deploy lorry loader at designated hoisting bay

- Shift out machine from production to loading area

- Rigger to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine and load onto lorry chassis

- Secured with strapping belt

- Transport to another building

- Unload, shift and position into factory premise

(2)

- Drive forklift into factory premise

- Assist customer engineer to remove bolting and machine leg

- Shift out to loading area

- Load up onto lorry chassis

- Secured with strapping belt

- Transport to another building

- Unload and position outside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175